Important Trading Note for the Americas:

- USA: Extremely strict on Sulfur (EPA regulations) and Vapor Pressure (RVP) for gasoline.

- Canada: Winter specs (Cloud Point/Pour Point) are critical due to climate.

- Mexico: Often aligns with US Gulf Coast (USGC) standards but check for specific CRE (Energy Regulatory Commission) requirements.

1. ULSD (Ultra Low Sulfur Diesel)

Standard: ASTM D975 (Grade No. 2-D S15)

Target Market: USA & Canada (On-road/Off-road standard).

This is the standard "clean" diesel required for modern engines in North America.

| Parameter | Test Method | Typical Limit |
| --- | --- | --- |
| Sulfur Content | ASTM D5453 | Max 15 ppm (mg/kg) |
| Cetane Index | ASTM D976 | Min 40 (often 42-45 typical) |
| Flash Point | ASTM D93 | Min 52°C (125°F) |
| Distillation (90% Recovery) | ASTM D86 | Min 282°C / Max 338°C |
| Viscosity @ 40°C | ASTM D445 | 1.9 – 4.1 cSt |
| Water & Sediment | ASTM D2709 | Max 0.05% vol |

2. EN590 (Automotive Diesel)

Standard: CEN EN 590 (European Standard)

Target Market: Global standard, but for the Americas, this is often imported as "10ppm Diesel" and re-certified to ASTM D975.

Note: EN590 generally implies a higher Cetane number (51) compared to the basic US spec (40), making it a premium product in the US market.

| Parameter | Test Method | Typical Limit |
| --- | --- | --- |
| Sulfur Content | ISO 20884 | Max 10 ppm |
| Cetane Number | EN ISO 5165 | Min 51 |
| Density @ 15°C | EN ISO 12185 | 820 – 845 kg/m³ |
| Flash Point | EN ISO 2719 | Min 55°C |
| Polycyclic Aromatic Hydrocarbons | EN 12916 | Max 8.0% m/m |
| Water Content | EN ISO 12937 | Max 200 mg/kg |

3. VLSFO (Very Low Sulfur Fuel Oil)

Standard: ISO 8217:2017 (Grade RMG 380 or RME 180)

Target Market: Maritime/Bunkering (IMO 2020 Compliant).

| Parameter | Test Method | Typical Limit |
| --- | --- | --- |
| Sulfur | ISO 8754 | Max 0.50% (Strict IMO limit) |
| Viscosity @ 50°C | ISO 3104 | Max 380 cSt (usually lower in reality) |
| Flash Point | ISO 2719 | Min 60°C |
| Pour Point | ISO 3016 | Max 30°C (Winter) |
| Aluminum + Silicon | ISO 10478 | Max 60 mg/kg (Cat fines) |
| Water | ISO 3733 | Max 0.5% vol |

3. Naphtha

Standard: No single fixed "grade." Usually sold as Light, Heavy, or Full Range.

Target Market: Petrochemical feedstocks (diluent for heavy crude in Canada/Mexico).

| Parameter | Test Method | Typical (Full Range) |
| --- | --- | --- |
| Density @ 15°C | ASTM D4052 | 0.65 – 0.75 g/cm³ |
| Initial Boiling Point (IBP) | ASTM D86 | ~35°C |
| Final Boiling Point (FBP) | ASTM D86 | ~180°C - 200°C |
| Sulfur Content | ASTM D5453 | Max 500 ppm (varies widely) |
| P.O.N.A. | ASTM D6730 | Report % (Paraffins, Olefins, Naphthenes, Aromatics) |
| Color | Saybolt | Min +25 (Water White) |

4. Jet Fuel A-1

Standard: ASTM D1655 (Specification for Aviation Turbine Fuels) or DEF STAN 91-091.

Target Market: International Aviation (Canada/Mexico).

Note: The USA uses "Jet A" primarily, which has a higher freezing point (-40°C) compared to "Jet A-1" (-47°C). A-1 is preferred for international routes.

| Parameter | Test Method | Typical Limit |
| --- | --- | --- |
| Freezing Point | ASTM D2386 | Max -47°C (Critical for Canada) |
| Flash Point | ASTM D56/D93 | Min 38°C |
| Density @ 15°C | ASTM D4052 | 775 – 840 kg/m³ |
| Sulfur (Total) | ASTM D4294 | Max 0.30% mass |
| Smoke Point | ASTM D1322 | Min 25 mm |
| Thermal Stability (JFTOT) | ASTM D3241 | Pass (<260°C) |

5. LPG (Liquefied Petroleum Gas)

Standard: ASTM D1835

Target Market: Heating, cooking, and transport. In the Americas, "Propane HD-5" is the common grade.

| Parameter | Test Method | Typical (Propane HD-5) |
| --- | --- | --- |
| Vapor Pressure @ 37.8°C | ASTM D1267 | Max 208 psig |
| Propane Content | ASTM D2163 | Min 90% (often >95%) |
| Butane and heavier | ASTM D2163 | Max 2.5% |
| Sulfur | ASTM D2784 | Max 123 ppm (low) |
| Residue (Evaporation) | ASTM D2158 | Max 0.05 ml |
| Corrosion (Copper Strip) | ASTM D1838 | No. 1 |